# CCNA: Chapter One: Introduction to TCP/IP Network

Adjacent-layer Interaction: Occurs on one computer, with two adjacent layers in the networking model working together. The higher layer requests services from the lower layer, and the lower layer provides the services to the next higher layer.

- Example: The process of HTTP asking TCP to send some data and making sure that it is received correctly.

Same-layer Interaction: When two computers use a protocol to communicate with the same layer on another computer. The protocol defines a header that communicates what each computer wants to do.

- The two devices set values in the header, send the header and the encapsulated data, with the receiving devices interpreting the header to decide what action to take.

De-encapsulation: The process used by a receiving device to remove one or more of the protocol headers. Data is de-encapsulated when moving up the stack toward the end-user application.

- A computer would receive data over the network in which they interpret the lower-layer header. When finished with each header, the computer removes the header to reveal the next higher-layer PDU.

Encapsulation: The process of adding headers & trailers around some data. Each layer adds its own header to the data supplied by the higher layer.

- The placement of data from a higher-layer protocol behind the header of the next-lower-layer protocol. IP Packet can be encapsulated in an Ethernet header and trailer before being sent over an Ethernet.

Protocol Data Unit (PDU): A PDU Represents the bits that include the headers & trailer for that layer; it also includes encapsulated data. PDU does not include any lower-layer headers & trailers

- A generic term that refers to the header defined by the specific layer on the networking model. It also defines the data that’s encapsulated by the header of that layer; does not include any lower-layer headers and trailers

Frame: Datalink (Layer 2) PDU. A Frame is used to identify an entity created when encapsulating data inside data-link layer headers & trailers. Each frame includes Header, Data, & Trailer.

- A term referring to a data-link header and trailer, plus the data encapsulated between the header and trailer.

Packet: Network (Layer 3) PDU. A Packet is a logical grouping of bytes that include the network layer header & encapsulated data. A packet doesn’t include any headers & trailers below the network layer.

- The network layer forwards the data from sending host to destination host

Segment: Transport (Layer 4) PDU. A term used to describe a TCP header and the encapsulated data. The process of accepting large blocks of data from the Application Layer and breaking it into smaller pieces of data to fit into TCP segments. The term refers to a TCP Header & the encapsulated data within it.

- The transport layer divides the data into smaller blocks (i.e., segments or datagrams) that make it easier to manage & transport the data. In Ethernet, a segment is either a single Ethernet cable or a single collision domain

Networking Model: An architectural blueprint that refers to a comprehensive set of documents that define everything that should happen for a computer network to work. These documents define protocols and standards that allow all the devices on a network to communicate.

- Generic term referring to any set of protocols and standards collected into a comprehensive grouping that allows all the devices to communicate. Examples include the TCP/IP and OSI Model